WeAct Studio STM32H723VGT6

Custom ArduPilot Flight Controller

Complete Build Guide — Open Source

| Parameter | Value |
| --- | --- |
| ArduPilot Version | ArduCopter V4.7.0-dev |
| MCU | STM32H723VGT6 (Cortex-M7 @ 400MHz) |
| Flash | 1MB internal (640KB firmware + 256KB storage) |
| RAM | 320KB AXI SRAM @ 0x24000000 |
| IMU | MPU9250 on SPI1 |
| Barometer | BMP280 on I2C1 |
| USB | OTG_HS Full-Speed, CDC ACM (ttyACM0) |
| Motor Outputs | 8x PWM (TIM1/TIM2/TIM3/TIM4/TIM8) |
| Serial Ports | USB + 2x UART + 1x UART RC |
| SD Card | SDMMC1 (onboard MicroSD slot) |
| External Compass | I2C2 on PB10/PB11 |
| Guide Version | 1.1 — March 2026 |

⚠ Developer build only. Not certified for safety-critical applications.

# 1. Overview

This guide documents the complete process of building a custom ArduPilot flight controller using the WeAct Studio STM32H723VGT6 development board. The entire project is open-source — anyone with the same hardware can follow this guide to build a working quadcopter flight controller from a $10 developer board.

The WeAct STM32H723VGT6 is a compact Cortex-M7 board featuring an onboard 0.96" ST7735 TFT display, MicroSD card slot, 8MB SPI flash, and camera interface — making it unusually feature-rich for its price. While ArduPilot does not have a native driver for the ST7735 display, the SD card and all GPIO peripherals work fully.

⚠ SPI1 conflict: The WeAct board has an onboard 8MB SPI flash chip on PA5/PA6/PA7 (same as SPI1 IMU pins). Disconnect or disable the onboard flash (SB3 solder bridge) when using an external IMU on SPI1.

## 1.1 What Works

- USB-C MAVLink connection (Mission Planner, QGroundControl)

- IMU: MPU9250 accelerometer + gyroscope

- Barometer: BMP280 altitude sensing

- 8x PWM motor outputs (quadcopter to octocopter)

- GPS via UART1 (PA9/PA10)

- Telemetry radio via UART2 (PA2/PA3)

- RC Receiver (ELRS/SBUS) via UART4 (PB8/PB9)

- Battery voltage/current monitoring via ADC

- SD card data logging via SDMMC1

- External I2C compass via I2C2 (PB10/PB11)

## 1.2 Known Limitations

- HAL_LOGGING_ENABLED=0 — SD card present but logging disabled (flash size)

- No onboard compass — AK8963 inside MPU9250 not detected; use external

- ST7735 onboard display — no ArduPilot driver; use I2C OLED instead

- EKF2 and EKF3 disabled to fit in 640KB firmware region

- Scripting (Lua) disabled

# 2. Hardware & Wiring

## 2.1 Bill of Materials

| Component | Model | Qty | Notes |
| --- | --- | --- | --- |
| Flight Controller | WeAct STM32H723VGT6 | 1 | Main board |
| IMU | MPU9250 module | 1 | SPI, 3.3V |
| Barometer | BMP280 module | 1 | I2C, addr 0x76 |
| GPS Module | u-blox M8N or M10 | 1 | UART, 9600 baud |
| RC Receiver | ELRS or SBUS receiver | 1 | UART or SBUS |
| ESCs | BLHeli_S or AM32 | 4 | PWM or DSHOT |
| ST-Link V2 | ST-Link V2 clone | 1 | For debugging |
| USB Cable | USB-C data cable | 1 | Data, not charge-only |
| Compass (optional) | QMC5883L I2C module | 1 | Connect to I2C2 |

## 2.2 Complete Pin Assignment

| STM32 Pin | Function | Peripheral | Notes |
| --- | --- | --- | --- |
| PA4 | SPI1_CS | MPU9250 NCS | Chip select |
| PA5 | SPI1_SCK | MPU9250 SCK | SPI clock |
| PA6 | SPI1_MISO | MPU9250 SDO | SPI MISO |
| PA7 | SPI1_MOSI | MPU9250 SDI | SPI MOSI |
| PB0 | MPU9250_DRDY | MPU9250 INT | Data ready |
| PB6 | I2C1_SCL | BMP280 SCL | Shared I2C bus |
| PB7 | I2C1_SDA | BMP280 SDA | Shared I2C bus |
| PB10 | I2C2_SCL | Ext. Compass SCL | External compass |
| PB11 | I2C2_SDA | Ext. Compass SDA | External compass |
| PA9 | USART1_TX | GPS RX | Serial1 |
| PA10 | USART1_RX | GPS TX | Serial1 |
| PA2 | USART2_TX | Telemetry RX | Serial2 |
| PA3 | USART2_RX | Telemetry TX | Serial2 |
| PB8 | UART4_RX | RC Receiver TX | Serial3 RC |
| PB9 | UART4_TX | RC Receiver RX | Serial3 RC |
| PA11 | OTG_FS_DM | USB-C D- | USB |
| PA12 | OTG_FS_DP | USB-C D+ | USB |
| PC0 | ADC1_IN10 | Battery Voltage | Via divider |
| PC1 | ADC1_IN11 | Battery Current | Via sensor |
| PA0 | TIM2_CH1 | Motor 1 ESC | PWM |
| PA1 | TIM2_CH2 | Motor 2 ESC | PWM |
| PA8 | TIM1_CH1 | Motor 3 ESC | PWM |
| PB1 | TIM3_CH4 | Motor 4 ESC | PWM |
| PC6 | TIM8_CH1 | Motor 5 ESC | PWM |
| PC7 | TIM8_CH2 | Motor 6 ESC | PWM |
| PD12 | TIM4_CH1 | Motor 7 ESC | PWM |
| PD13 | TIM4_CH2 | Motor 8 ESC | PWM |
| PC8 | SDMMC1_D0 | SD Card | Onboard slot |
| PC9 | SDMMC1_D1 | SD Card | Onboard slot |
| PC10 | SDMMC1_D2 | SD Card | Onboard slot |
| PC11 | SDMMC1_D3 | SD Card | Onboard slot |
| PC12 | SDMMC1_CK | SD Card | Onboard slot |
| PD2 | SDMMC1_CMD | SD Card | Onboard slot |
| PA13 | SWDIO | ST-Link | Debug only |
| PA14 | SWDCLK | ST-Link | Debug only |

## 2.3 RC Receiver Wiring

| Receiver Type | Connection | Mission Planner Setting |
| --- | --- | --- |
| ExpressLRS (ELRS) | TX→PB8, RX→PB9, GND, 3.3V | SERIAL3_PROTOCOL=23, SERIAL3_BAUD=420 |
| SBUS | SBUS Out→PB8, GND, 5V | SERIAL3_PROTOCOL=23, SERIAL3_OPTIONS=3 |
| DSM/Spektrum | Data→PB8, GND, 3.3V | SERIAL3_PROTOCOL=23 |
| CRSF/Crossfire | TX→PB8, RX→PB9, GND, 5V | SERIAL3_PROTOCOL=23, SERIAL3_BAUD=416 |

## 2.4 External Compass (Optional)

Connect any I2C compass to I2C2 (PB10=SCL, PB11=SDA). Recommended modules:

| Compass | I2C Address | Part Number |
| --- | --- | --- |
| QMC5883L | 0x0D | Cheap clone HMC5883L replacement |
| HMC5883L | 0x1E | Original Honeywell (discontinued) |
| IST8310 | 0x0C | Used in many GPS modules |
| LIS3MDL | 0x1C or 0x1E | ST Microelectronics |

# 3. Build Environment

## 3.1 System Requirements

- Ubuntu 22.04 or 24.04 LTS (recommended)

- arm-none-eabi-gcc 13.x

- ~10GB free disk space

- Python 3.9+

## 3.2 Setup Commands

sudo apt-get update && sudo apt-get install -y \

git python3 python3-pip python3-venv \

arm-none-eabi-gcc arm-none-eabi-g++ \

arm-none-eabi-binutils dfu-util openocd

git clone https://github.com/ArduPilot/ardupilot.git

cd ardupilot

git submodule update --init --recursive

python3 -m venv venv-ardupilot

source venv-ardupilot/bin/activate

pip install empy==3.3.4 pexpect future pymavlink MAVProxy

# 4. Board Definition Files

## 4.1 Create Directory

mkdir -p ~/ardupilot/libraries/AP_HAL_ChibiOS/hwdef/WeActH723

## 4.2 Complete hwdef.dat

Create this file at: ~/ardupilot/libraries/AP_HAL_ChibiOS/hwdef/WeActH723/hwdef.dat

# WeAct Studio STM32H723VGT6 ArduPilot Flight Controller

# https://github.com/WeActStudio/WeActStudio.MiniSTM32H723

MCU STM32H7xx STM32H723xx

OSCILLATOR_HZ 25000000

FLASH_SIZE_KB 1024

APJ_BOARD_ID 1023

# Flash layout: Sector0=BL, Sectors1-5=FW, Sectors6-7=Storage

FLASH_RESERVE_START_KB 128

STORAGE_FLASH_PAGE 6

define HAL_STORAGE_SIZE 16384

# USB (OTG_HS in Full-Speed mode, AF10)

PA11 OTG_FS_DM OTG1

PA12 OTG_FS_DP OTG1

define BOARD_OTG_NOVBUSSENS 1

# Serial ports

PA9  USART1_TX USART1          # Serial1 = GPS

PA10 USART1_RX USART1

PA2  USART2_TX USART2          # Serial2 = Telemetry

PA3  USART2_RX USART2

PB8  UART4_RX UART4 NODMA      # Serial3 = RC Receiver

PB9  UART4_TX UART4 NODMA

SERIAL_ORDER OTG1 USART1 USART2 UART4

# SPI1 - MPU9250 IMU

PA5 SPI1_SCK SPI1

PA6 SPI1_MISO SPI1

PA7 SPI1_MOSI SPI1

PA4 MPU9250_CS CS

PB0 MPU9250_DRDY INPUT

SPIDEV mpu9250 SPI1 DEVID1 MPU9250_CS MODE3 2*MHZ 8*MHZ

IMU Invensense SPI:mpu9250 ROTATION_NONE

# I2C1 - BMP280 Barometer

PB6 I2C1_SCL I2C1

PB7 I2C1_SDA I2C1

I2C_ORDER I2C1 I2C2

BARO BMP280 I2C:0:0x76

# I2C2 - External Compass (PB10=SCL, PB11=SDA)

PB10 I2C2_SCL I2C2

PB11 I2C2_SDA I2C2

define HAL_PROBE_EXTERNAL_I2C_COMPASSES 1

define AP_COMPASS_AK8963_ENABLED 1

define AP_COMPASS_HMC5843_ENABLED 1

define AP_COMPASS_QMC5883L_ENABLED 1

define AP_COMPASS_IST8310_ENABLED 1

# ADC - Battery monitoring

PC0 BATT_VOLTAGE_SENS ADC1 SCALE(1)

PC1 BATT_CURRENT_SENS ADC1 SCALE(1)

# PWM Motor outputs

PA0  TIM2_CH1 TIM2 PWM(1) GPIO(50)   # Motor 1

PA1  TIM2_CH2 TIM2 PWM(2) GPIO(51)   # Motor 2

PA8  TIM1_CH1 TIM1 PWM(3) GPIO(52)   # Motor 3

PB1  TIM3_CH4 TIM3 PWM(4) GPIO(53)   # Motor 4

PC6  TIM8_CH1 TIM8 PWM(5) GPIO(54)   # Motor 5

PC7  TIM8_CH2 TIM8 PWM(6) GPIO(55)   # Motor 6

PD12 TIM4_CH1 TIM4 PWM(7) GPIO(56)   # Motor 7

PD13 TIM4_CH2 TIM4 PWM(8) GPIO(57)   # Motor 8

# SD Card - SDMMC1 (onboard MicroSD)

PC8  SDMMC1_D0  SDMMC1

PC9  SDMMC1_D1  SDMMC1

PC10 SDMMC1_D2  SDMMC1

PC11 SDMMC1_D3  SDMMC1

PC12 SDMMC1_CK  SDMMC1

PD2  SDMMC1_CMD SDMMC1

define HAL_OS_FATFS_IO 1

# Firmware validation descriptor

define AP_CHECK_FIRMWARE_ENABLED 1

# 1MB flash size minimization

include ../include/minimize_features.inc

define HAL_NAVEKF2_AVAILABLE 0

define HAL_NAVEKF3_AVAILABLE 0

define AP_SCRIPTING_ENABLED 0

define HAL_LOGGING_ENABLED 0

define AP_COMPASS_BACKEND_DEFAULT_ENABLED 0

define AP_COMPASS_EXTERNALAHRS_ENABLED 0

define HAL_EXTERNAL_AHRS_ENABLED 0

define AP_VOLZ_ENABLED 0

define HAL_SUPPORT_RCOUTPUT_SERIAL 0

define AP_MOTORS_FRAME_DEFAULT_ENABLED 0

define AP_MOTORS_FRAME_QUAD_ENABLED 1

define AP_MOTORS_FRAME_HEXA_ENABLED 1

define AP_MOTORS_FRAME_OCTA_ENABLED 1

define HAL_RALLY_ENABLED 0

define AP_ICENGINE_ENABLED 0

define AP_ADVANCEDFAILSAFE_ENABLED 0

define AC_PRECLAND_ENABLED 0

define AP_SBUSOUTPUT_ENABLED 0

define HAL_PARACHUTE_ENABLED 0

define HAL_BUTTON_ENABLED 0

define AP_OPTICALFLOW_ENABLED 0

define MODE_ACRO_ENABLED 0

define MODE_DRIFT_ENABLED 0

define MODE_FLIP_ENABLED 0

define HAL_SPEKTRUM_TELEM_ENABLED 0

define HAL_CRSF_TELEM_ENABLED 0

## 4.3 hwdef-bl.dat (Bootloader)

Create at: ~/ardupilot/libraries/AP_HAL_ChibiOS/hwdef/WeActH723/hwdef-bl.dat

include ../include/minimize_common.inc

MCU STM32H7xx STM32H723xx

OSCILLATOR_HZ 25000000

FLASH_SIZE_KB 1024

APJ_BOARD_ID 1023

FLASH_BOOTLOADER_LOAD_KB 128

PA11 OTG_FS_DM OTG1

PA12 OTG_FS_DP OTG1

define BOARD_OTG_NOVBUSSENS 1

# 5. Required Source Code Patches

These patches fix STM32H723-specific differences from H743. Apply all patches before building.

## 5.1 RAM Map — STM32H723xx.py

File: libraries/AP_HAL_ChibiOS/hwdef/scripts/STM32H723xx.py

Problem: Default RAM map uses DTCM (0x20000000) first. Bootloader expects AXI SRAM (0x24000000) → hard fault on jump.

# CHANGE: Reorder RAM_MAP — AXI SRAM must be first

'RAM_MAP' : [

(0x24000000, 320, 4),  # AXI SRAM — PRIMARY (was second)

(0x20000000, 128, 2),  # DTCM — secondary

(0x30000000,  32, 8),

(0x38000000,  16, 1),

]

## 5.2 USB Pin AF — STM32H723xx.py

Problem: OTG_HS pins assigned AF0, need AF10 for USB operation.

# CHANGE: Fix alternate function for USB pins

"PA11:OTG_HS_DM"  :  10,   # was 0

"PA12:OTG_HS_DP"  :  10,   # was 0

"PA11:OTG_FS_DM"  :  10,   # add new entry

"PA12:OTG_FS_DP"  :  10,   # add new entry

## 5.3 25MHz PLL — stm32h7_type2_mcuconf.h

File: libraries/AP_HAL_ChibiOS/hwdef/common/stm32h7_type2_mcuconf.h

Problem: No PLL config for 25MHz HSE crystal. Add before the #else #error line:

#elif STM32_HSECLK == 25000000

// 25MHz HSE → 400MHz SYSCLK (PLL1: /5=5MHz, x80=400MHz)

#define STM32_PLL1_DIVM_VALUE  5

#define STM32_PLL1_DIVN_VALUE  80

#define STM32_PLL1_DIVP_VALUE  1

#define STM32_PLL1_DIVQ_VALUE  8

#define STM32_PLL1_DIVR_VALUE  2

#define STM32_PLL2_DIVM_VALUE  5

#define STM32_PLL2_DIVN_VALUE  128

#define STM32_PLL2_DIVP_VALUE  3

#define STM32_PLL2_DIVQ_VALUE  8

#define STM32_PLL2_DIVR_VALUE  2

#define STM32_PLL3_DIVM_VALUE  5

#define STM32_PLL3_DIVN_VALUE  76

#define STM32_PLL3_DIVP_VALUE  2

#define STM32_PLL3_DIVQ_VALUE  2

#define STM32_PLL3_DIVR_VALUE  4

## 5.4 ADC3 Guard — stm32h7_type2_mcuconf.h

Problem: H723 has no ADC3. Guard all ADC3 config:

#ifndef STM32H723_MCUCONF

#define STM32_ADC_ADC3_DMA_PRIORITY  2

#define STM32_ADC_ADC3_IRQ_PRIORITY  5

#endif

#if !defined(STM32H723_MCUCONF) && !defined(STM32_ADC_ADC3_CLOCK_MODE)

#define STM32_ADC_ADC3_CLOCK_MODE  ADC_CCR_CKMODE_AHB_DIV4

#endif

#ifndef STM32_ADC_ADC12_DMA_STREAM

#define STM32_ADC_ADC12_DMA_STREAM STM32_ADC_ADC1_DMA_STREAM

#endif

## 5.5 ADCD3 Guard — AnalogIn.cpp

File: libraries/AP_HAL_ChibiOS/AnalogIn.cpp

Problem: Code references ADCD3 which does not exist on H723. Wrap all 5 references:

// Line ~402:

#ifdef ADCD3

if (adcp == &ADCD3) {

#else

if (false) {

#endif

// Line ~468:

#ifdef ADCD3

adcp = &ADCD3;

#endif

// Lines ~525-527:

#ifdef ADCD3

adcSTM32EnableVREF(&ADCD3);

adcSTM32EnableTS(&ADCD3);

adcSTM32EnableVBAT(&ADCD3);

#endif

## 5.6 strlen Fix — AP_Bootloader/support.cpp

Problem: GCC 13.2 optimizes the custom strlen() into an infinite loop (b.w strlen).

// BEFORE:

size_t strlen(const char *s1)

// AFTER:

__attribute__((optimize("O0"))) size_t strlen(const char *s1)

## 5.7 USB Reset + CRS — board.c

File: libraries/AP_HAL_ChibiOS/hwdef/common/board.c

Add to boardInit() function — resets USB peripheral after bootloader and enables Clock Recovery System for stable HSI48:

#if defined(STM32H723xx) || defined(STM32H7xx)

// Reset USB OTG_HS (clear bootloader state)

RCC->AHB1RSTR |= RCC_AHB1RSTR_USB1OTGHSRST;

volatile uint32_t dummy = RCC->AHB1RSTR; (void)dummy;

RCC->AHB1RSTR &= ~RCC_AHB1RSTR_USB1OTGHSRST;

// Enable CRS: sync HSI48 to USB SOF for stable enumeration

RCC->APB1HENR |= RCC_APB1HENR_CRSEN;

CRS->CFGR = (2U << 28);  // SYNCSRC = USB SOF

CRS->CR |= CRS_CR_AUTOTRIMEN | CRS_CR_CEN;

#endif

## 5.8 USB Turnaround — hal_usb_lld.c

File: modules/ChibiOS/os/hal/ports/STM32/LLD/OTGv1/hal_usb_lld.c

Problem: TRDT_VALUE_FS=5 causes enumeration failure on H7 at 200MHz AHB.

#define TRDT_VALUE_FS  9   // was 5

# 6. Build & Flash

## 6.1 Build Bootloader

cd ~/ardupilot

./waf configure --board WeActH723 --bootloader

./waf bootloader

cp build/WeActH723/bin/AP_Bootloader.bin Tools/bootloaders/WeActH723_bl.bin

## 6.2 Build Firmware

cd ~/ardupilot

./waf distclean

./waf configure --board WeActH723

./waf copter

# Output files:

# build/WeActH723/bin/arducopter.bin      raw binary

# build/WeActH723/bin/arducopter.apj      ArduPilot package

# build/WeActH723/bin/arducopter_with_bl.hex  combined hex

## 6.3 Flash via dfu-util (Linux)

# Step 1: Hold BOOT0, plug USB → DFU mode

lsusb | grep '0483:df11'   # verify DFU

# Step 2: Flash bootloader

sudo dfu-util -a 0 --dfuse-address 0x08000000 \

-D ~/ardupilot/build/WeActH723/bin/AP_Bootloader.bin

# Step 3: Hold BOOT0, plug USB again → DFU mode

# Step 4: Flash firmware

sudo dfu-util -a 0 --dfuse-address 0x08020000:leave \

-D ~/ardupilot/build/WeActH723/bin/arducopter.bin

# Step 5: Replug WITHOUT BOOT0

sleep 5 && lsusb | grep '1209:5741'  # should show ArduPilot

## 6.4 Flash via STM32CubeProgrammer (Windows)

- Connect USB-C while holding BOOT0 button

- Open STM32CubeProgrammer → USB → Connect

- Download tab → browse to arducopter_with_bl.hex

- Click Start Programming (no address needed for .hex)

- Replug without BOOT0 — board enumerates as ArduPilot

## 6.5 Update Firmware (After Initial Flash)

# Using ArduPilot uploader (Linux):

python3 ~/ardupilot/Tools/scripts/uploader.py \

--port /dev/ttyACM0 \

~/ardupilot/build/WeActH723/bin/arducopter.apj

# Or via Mission Planner:

# Config → Install Firmware → Load custom firmware → .apj

# 7. Flash Memory Layout

| Sector | Address | Size | Contents |
| --- | --- | --- | --- |
| 0 | 0x08000000 | 128 KB | ArduPilot Bootloader |
| 1 | 0x08020000 | 128 KB | Firmware start |
| 2 | 0x08040000 | 128 KB | Firmware |
| 3 | 0x08060000 | 128 KB | Firmware |
| 4 | 0x08080000 | 128 KB | Firmware |
| 5 | 0x080A0000 | 128 KB | Firmware end (~740KB total) |
| 6 | 0x080C0000 | 128 KB | Parameter storage (page 1) |
| 7 | 0x080E0000 | 128 KB | Parameter storage (page 2) |

⚠ Sectors 6+7 are reserved for parameter storage (STORAGE_FLASH_PAGE 6). Do NOT write firmware beyond sector 5 or parameters will be erased.

# 8. Mission Planner Configuration

## 8.1 Initial Connection

- Connect USB-C — board boots, ttyACM0 / COM port appears

- Open Mission Planner → select COM port → 115200 baud → Connect

- Go to CONFIG → Full Parameter List → verify BARO1_DEVID=226817

## 8.2 Serial Port Configuration

| Parameter | Value | Purpose |
| --- | --- | --- |
| SERIAL0_PROTOCOL | 2 | USB = MAVLink2 |
| SERIAL1_PROTOCOL | 5 | GPS on USART1 |
| SERIAL1_BAUD | 38 | 9600 baud GPS |
| SERIAL2_PROTOCOL | 2 | Telemetry MAVLink |
| SERIAL2_BAUD | 57 | 57600 baud |
| SERIAL3_PROTOCOL | 23 | RC Input on UART4 |
| SERIAL3_OPTIONS | 0 (ELRS) or 3 (SBUS) | SBUS needs invert |

## 8.3 Motor Outputs (X-Frame Quadcopter)

| Output | STM32 Pin | Motor Position | Spin Direction |
| --- | --- | --- | --- |
| 1 | PA0 (TIM2_CH1) | Front-Right | CCW |
| 2 | PA1 (TIM2_CH2) | Rear-Left | CCW |
| 3 | PA8 (TIM1_CH1) | Front-Left | CW |
| 4 | PB1 (TIM3_CH4) | Rear-Right | CW |

## 8.4 Mandatory Calibration Sequence

- Accelerometer Calibration (Setup → Accel Calibration)

- Compass Calibration — skip if no external compass fitted

- Radio Calibration (Setup → Radio Calibration)

- ESC Calibration (Setup → ESC Calibration)

- Flight Mode Setup — minimum: Stabilize + AltHold + Loiter

- Failsafe Setup — RTL on signal loss

- Motor Test — verify correct spin directions

# 9. ST-Link V2 Debug Connection

## 9.1 SWD Wiring

| ST-Link V2 | WeAct Board | Function |
| --- | --- | --- |
| SWDIO | PA13 (SWDIO header) | Serial Wire Data |
| SWDCLK | PA14 (SWDCLK header) | Serial Wire Clock |
| GND | GND | Ground |
| 3.3V | 3.3V (optional) | Power source |

⚠ Firmware reconfigures PA13/PA14 at startup, disconnecting ST-Link. Hold BOOT0 when connecting to debug the bootloader only.

## 9.2 OpenOCD + GDB

# Terminal 1

openocd -f interface/stlink.cfg -f target/stm32h7x.cfg

# Terminal 2

gdb-multiarch -batch \

-ex 'set pagination off' \

-ex 'file ~/ardupilot/build/WeActH723/bootloader/AP_Bootloader' \

-ex 'target remote :3333' \

-ex 'monitor reset halt' \

-ex 'info registers pc sp' \

-ex 'backtrace 20' \

-ex 'quit'

# 10. Verification Checklist

| Test | Expected Result | Done? |
| --- | --- | --- |
| DFU mode (hold BOOT0+plug) | lsusb: 0483:df11 STM Device in DFU Mode | [ ] |
| Bootloader USB | lsusb: 1209:5741 / ttyACM0 appears as WeActH723-BL | [ ] |
| Firmware USB | Mission Planner connects, ArduCopter title bar | [ ] |
| Barometer | BARO1_DEVID = 226817 (BMP280 on I2C) | [ ] |
| IMU Accelerometer | INS_ACC_ID shows ACC_MPU6000/9250 on SPI1 | [ ] |
| IMU Gyroscope | INS_GYR_ID shows GYR_MPU6000/9250 on SPI1 | [ ] |
| Flash storage | No 'unable to init flash storage' in messages | [ ] |
| Motor outputs | 8 outputs visible in Mission Planner Servo Output | [ ] |
| RC Input | RC calibration shows stick inputs | [ ] |
| GPS | GPS lock achieved outdoors | [ ] |
| SD Card | microSD inserted, FAT32 formatted | [ ] |

# 11. Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| No USB after flash | Wrong flash address used | Flash bootloader to 0x08000000, firmware to 0x08020000 |
| Bootloader loops, no firmware jump | No app descriptor in firmware | Add define AP_CHECK_FIRMWARE_ENABLED 1 |
| USB error -110 / -71 | HSI48 not stable | Ensure CRS init in boardInit() is present |
| Hard fault on boot | Wrong RAM map (0x20000000 vs 0x24000000) | Fix STM32H723xx.py RAM_MAP — AXI SRAM first |
| strlen infinite loop | GCC 13 optimizer bug | Add __attribute__((optimize("O0"))) to strlen in support.cpp |
| IMU not detected | Wrong CS pin | CS must be PA4 not PB0 |
| Linker overflow | Firmware > 640KB | Disable more features in hwdef.dat |
| USB stable only with ST-Link | Power noise from PC USB port | Use powered USB hub or add 100uF capacitor on 3.3V |

# 12. References & Links

- WeAct Studio H723 Board: https://github.com/WeActStudio/WeActStudio.MiniSTM32H723

- ArduPilot Documentation: https://ardupilot.org/copter/

- ArduPilot Custom Boards Guide: https://ardupilot.org/dev/docs/building-the-code.html

- STM32H723 Reference Manual: RM0468 Rev 4 (st.com)

- ChibiOS RTOS: https://www.chibios.org/

- MicroPython H723 Reference: https://github.com/jkorte-dev/micropython-board-STM32H723VGT6

- dfu-util: http://dfu-util.sourceforge.net/

- STM32CubeProgrammer: https://www.st.com/en/development-tools/stm32cubeprog.html

WeActH723 ArduPilot Flight Controller Guide  |  v1.1  |  March 2026  |  MIT License  |  Open Source